# Technical Meeting Minutes

## UPF Service Classification and Core-to-RAN Service Awareness

### 1. Meeting Objective

The meeting focused on understanding the existing traffic/service-classification capabilities in the UPF and determining how those capabilities could ultimately be extended into an end-to-end Core–RAN service-aware solution.

The broader objective is to exploit Ericsson's presence across both the Core and RAN to determine whether information already available in the Core can provide additional intelligence to the RAN and enable more differentiated treatment of user traffic. The discussion specifically positioned service recognition/service awareness as one potential differentiator.

The immediate meeting scope was deliberately narrower: understand what classification information the UPF can produce today before deciding exactly how the RAN should consume and act on that information.

# 2. High-Level End-to-End Concept

The proposed concept can be summarized as:

UE/Application Traffic → UPF Service Detection & Classification → Service Insight → RAN → RAN Scheduler / QoS Treatment → Improved User Experience

Today, multiple application flows associated with a subscriber may effectively receive treatment based primarily on the established bearer/session/QoS framework.

The proposed enhancement is to make the network more service aware.

For example, the UPF could distinguish traffic such as:

- YouTube

- Netflix

- Web browsing

- Social networking

- AI traffic

- Messaging

- Video streaming

- Video calling

- Audio calling

The UPF could then communicate relevant service information toward the RAN. The RAN could potentially use that information to make more granular scheduling or resource-management decisions rather than treating all traffic associated with a user identically.

# 3. Existing UPF Role

The UPF already processes user-plane traffic and performs considerably more functionality than simple packet forwarding.

The discussion described the UPF as receiving relatively undifferentiated traffic and then performing:

Packet Detection → Traffic Classification → Policy/Service Processing → Forwarding

Once traffic is classified, existing UPF capabilities may support actions such as:

- Traffic shaping

- Forwarding

- QoS enforcement

- Buffering

- Content filtering

- Service chaining

- ABR shaping

- NAT-related processing

- Other policy-driven enforcement

However, the focus of this meeting was primarily on the classification stage, rather than these downstream enforcement functions.

# 4. Relationship Between Control Plane and UPF Classification

The discussion explained that subscriber/session-specific rules are provided through the Core control-plane architecture.

Conceptually:

PCF / Policy → SMF → UPF

The UPF receives rules describing how traffic belonging to a particular subscriber/session should be detected and processed.

Packet Detection Rules (PDRs) can include information such as:

- Source interface

- UE IP address

- Tunnel identifier

- Network instance

- Flow/filter information

The UPF first determines which subscriber/session/flow a packet belongs to and matches it against the relevant rules. Classification capabilities can then determine the actual application or service associated with that flow.

A simplified processing model discussed in the meeting is:

Incoming Packet

↓

Identify Session / PDR

↓

Apply Packet/Flow Filters

↓

Classify Service/Application

↓

Apply Applicable Processing Chain

↓

Forward Packet

The service chain can subsequently include QoS enforcement, filtering, shaping and other functions.

# 5. UPF Service Classification Capability

A significant part of the meeting was dedicated to explaining how sophisticated the existing classification engine already is.

The UPF classification architecture uses a combination of mechanisms rather than relying on a single identifier.

The mechanisms discussed included:

Heuristics + packet inspection + signatures/patterns + protocol information + classification rules

The classification system can use inspection across protocol layers, including Layer 3 and Layer 4 information, as part of a larger toolbox.

The system maintains signatures and patterns representing known Internet/application behavior. These signatures are continuously developed in a separate environment and packaged into heuristic updates that can be delivered into the operational environment.

# 6. Application-Level and Sub-Application-Level Classification

An important technical capability discussed was that classification does not necessarily stop at identifying an application.

For example, identifying:

Application = Facebook

may not provide sufficient granularity.

A single application can contain multiple services and multiple IP flows. The classification engine may therefore distinguish:

Facebook → Messaging

Facebook → Video Streaming

Facebook → Video Call

Facebook → Audio Call

The meeting indicated that classification can operate down to IP-flow granularity, allowing different flows belonging to the same application/session to be classified differently.

This capability is particularly important for the proposed FirstNet use case because simply labeling something as "video" or "streaming" may not be sufficient to determine its importance.

# 7. AI Traffic Classification

A specific question was raised regarding whether the UPF could distinguish AI traffic from non-AI traffic.

The response was that AI-related application signatures are already being developed/delivered through the heuristic classification mechanism.

Examples mentioned included services such as OpenAI-related traffic and Copilot, with the objective of determining which AI/cloud application is associated with a flow.

This capability could eventually provide a foundation for broader AI-aware network treatment, although no specific AI traffic policy was finalized during this meeting.

# 8. Classification of Encrypted Traffic

Encryption was identified as an important challenge because increasingly more application traffic is encrypted.

The discussion differentiated between payload encryption and information that may remain observable in protocol headers or connection establishment.

Two protocol families were specifically discussed:

QUIC/UDP

and

TLS/TCP

The classification engine can use observable protocol characteristics and available handshake/header information. TLS ClientHello information was specifically discussed.

However, participants recognized that application providers increasingly encrypt additional protocol/header information. Therefore, encrypted traffic classification is an evolving problem, and the available visibility may change as protocols and privacy mechanisms evolve.

# 9. Machine Learning and Heuristic Signature Updates

The vendor described an offline/lab-based process used to improve classification.

Large amounts of Internet/application traffic can be captured and analyzed in the lab. Machine-learning techniques can assist in discovering new patterns/signatures and improving the heuristic files.

Conceptually:

Internet Traffic Capture

↓

Offline/Lab Analysis

↓

Machine-Learning / Pattern Analysis

↓

New or Updated Signatures

↓

Heuristic Package

↓

UPF Classification Engine

The classification library is continuously expanded because Internet applications and their traffic behavior change over time.

The discussion indicated that the classification capability recognizes a large number of applications and is continuously updated as new applications and signatures emerge. Updates can then be provided to AT&T periodically or on demand.

# 10. Proposed Core-to-RAN Service Awareness

The key innovation under investigation is not simply UPF classification, because much of that capability already exists.

The new work is:

How can the service insight already generated by the UPF be communicated to the RAN and used meaningfully?

This was repeatedly identified as the main open technical problem.

Conceptually:

UPF

Service = X

Application/Sub-application = Y

Traffic/Flow = Z

↓

Core-to-RAN Service Information

↓

RAN

↓

Scheduler / Resource Allocation / QoS Decision

The RAN might then use the information for more granular service differentiation or scheduling.

However, exactly what information should be communicated, how frequently it should be updated, and what RAN action should result remained unresolved.

# 11. Potential GTP-U Extension

A question was raised regarding whether the service-classification information could potentially be carried through a custom attribute/information element associated with GTP-U.

This was acknowledged as one high-level possibility.

However, the meeting clearly stated that this mechanism has not yet been implemented. The Core and RAN teams are still investigating how service-classification information should be communicated.

The following remain under discussion:

What information should be sent?

What information element or extension should carry it?

How should the information be transferred toward the RAN?

How should the RAN interpret and use it?

Therefore, the UPF classification capability should be distinguished from the proposed Core-to-RAN signaling mechanism: the former exists today; the latter remains a work in progress.

# 12. Multi-Vendor RAN Concern

One of the strongest architecture concerns raised was multi-vendor interoperability.

AT&T's Core may simultaneously serve different RAN vendors, including Ericsson and Nokia.

If proprietary service-awareness information is added toward the RAN, the solution must avoid creating interoperability or operational problems for non-Ericsson RAN nodes.

A proposed direction was to determine whether the Core could selectively enable the additional information only when the destination RAN supports it.

Conceptually:

UPF

↙ ↘

Ericsson RAN Other-vendor RAN

Service insight Standard behavior

enabled without proprietary information

The team agreed that this issue must be considered early in the architecture/design phase, rather than addressed after implementation.

# 13. Operational Example Demonstrating Multi-Vendor Risk

A previous service/QoS experience was cited as an example of why interoperability must be designed carefully.

A premium/turbo-type service worked as expected in the macro network, but when users moved into a different-vendor small-cell environment, the expected treatment was not recognized and the resulting user experience was problematic.

The lesson was that a service enhancement that works in one RAN environment cannot be assumed to work seamlessly across all RAN technologies/vendors.

The architecture therefore needs explicit consideration of:

- Ericsson RAN

- Nokia/other RAN

- Small cells

- Mobility between RAN types

- Roaming scenarios

- Selective feature activation

# 14. FirstNet as a Primary Candidate Use Case

FirstNet emerged as one of the strongest potential use cases.

The underlying problem is that a priority user can generate both mission-critical traffic and ordinary consumer traffic.

For example:

FirstNet User

→ Mission-critical application

→ YouTube

→ Facebook

→ Netflix

→ Other applications

The objective should therefore not simply be:

FirstNet subscriber = prioritize everything

Instead, the proposed concept is closer to:

Subscriber + Service/Application Context → Appropriate Treatment

This would potentially allow the network to preserve mission-critical treatment while treating non-mission-critical application flows differently.

# 15. Critical FirstNet Classification Requirement

A key clarification was made regarding video traffic.

YouTube must not automatically be treated as non-critical entertainment traffic.

First responders may use services such as:

- YouTube

- Facebook Live

- Instagram

- Other social/video platforms

for investigation or operational purposes.

Therefore, broad classifications such as:

Video → deprioritize

or

Streaming → deprioritize

would be too coarse.

The classification and policy logic must be able to distinguish specific applications and potentially specific sub-application behaviors.

# 16. FirstNet Use Case 1 — Down-Prioritization

One proposed use case is to identify selected entertainment applications for extended-primary/subscriber-paid users and potentially reduce their priority.

Examples discussed included:

- Netflix

- Amazon Prime

- Hulu

At the same time, applications such as YouTube or Facebook should not automatically receive this treatment because they may have legitimate public-safety/investigative uses.

The discussion also indicated that this policy would not initially target primary FirstNet subscribers; the proposed starting scope was extended-primary subscribers.

# 17. FirstNet Use Case 2 — Uplift / Mission-Critical Priority

The second proposed use case operates in the opposite direction.

A push-to-talk/LMR interworking gateway was described. This device provides interoperability between mobile devices and LMR radios, but the current architecture does not provide an easy mechanism for assigning the desired mission-critical QoS treatment to that equipment.

The proposed concept is:

Detect traffic/device associated with the LMR gateway

↓

Recognize mission-critical service

↓

Apply appropriate mission-critical priority/QoS

This represents the uplift use case, whereas selected entertainment traffic represents the downlift/down-prioritization use case.

These two cases were proposed as strong candidates for driving the end-to-end implementation and October demonstration.

# 18. VPN Challenge

VPN traffic was identified as another significant technical issue, particularly for FirstNet/public-safety environments.

Participants noted that substantial public-safety traffic may traverse VPNs.

The problem becomes:

Application

↓

VPN Encryption/Tunnel

↓

UPF

↓

How does UPF identify the original service?

The team acknowledged that classification may become significantly more limited when application traffic is encapsulated inside a VPN.

Therefore, VPN behavior must be explicitly evaluated as part of the FirstNet use-case design.

# 19. RAN-Side Questions Still Unresolved

The meeting repeatedly emphasized that understanding UPF classification is only the first half of the problem.

The RAN team must determine:

1. What information does the RAN actually need?

Application ID?

Service category?

Sub-application?

Priority indication?

Flow identifier?

QoS recommendation?

2. What granularity is practical?

Per UE?

Per PDU session?

Per QoS flow?

Per IP flow?

Per application?

Per sub-application?

3. How frequently must the information be updated?

Per packet would likely be unnecessary and expensive.

Classification may initially require inspection of the first packets of a flow; after classification, subsequent packets can use a faster processing path.

4. What does the RAN actually do with the information?

Potential actions discussed conceptually include:

- Scheduler differentiation

- Resource prioritization

- QoS differentiation

- More efficient radio-resource management

- Improved user experience

These actions still require detailed RAN-side architecture and validation.

# 20. Need for Traffic Baseline and Analytics

Before designing policy, the team proposed first understanding what traffic actually exists in the network.

The suggested approach was to use available analytics/EBM information to determine the traffic distribution.

For example:

Network Traffic

→ YouTube: X%

→ Meta: Y%

→ Netflix: Z%

→ AI traffic: A%

→ Other traffic: B%

The purpose is not merely reporting traffic percentages. The baseline would allow the team to identify where meaningful optimization opportunities exist and then design service-aware policies around real traffic patterns.

The team agreed that this traffic analysis should be an initial step before selecting broader optimization targets.

# 21. Monetization Discussion

Direct monetization was questioned.

The concern was that customers are unlikely to pay additional money simply because the internal Core/RAN interaction is more efficient.

Therefore, the initial value proposition may be more appropriately framed as:

Better service differentiation

More efficient radio-resource utilization

Better experience for traffic/users that legitimately require priority

rather than immediate direct monetization.

Indirect monetization may become possible if service-aware treatment becomes part of a premium differentiated service. However, the meeting did not reach a final monetization conclusion.

# 22. September Demonstration

The near-term demonstration is intended primarily to show the existing Core/UPF capability.

The proposed demonstration flow is:

Generate simulated/test UE traffic

↓

UPF receives traffic

↓

Classification engine identifies applications/services

↓

EA/analytics obtains classification information

↓

Dashboard displays classified traffic

Example applications mentioned included YouTube and Netflix.

Importantly, the team clarified that this demonstration is Core-focused and does not yet include RAN actions.

The purpose is to ensure that all stakeholders first understand what capabilities already exist in the UPF before moving to the end-to-end design.

# 23. October End-to-End Demonstration Target

The subsequent target is significantly more ambitious.

The October demonstration is expected to move beyond simply showing:

"The UPF can identify the application."

Instead, the target is to demonstrate:

UPF Classification

↓

Service Information communicated toward RAN

↓

RAN recognizes differentiated service

↓

Different treatment is applied

↓

Observable end-to-end behavior

The two FirstNet use cases—one uplift and one down-prioritization—were proposed as practical starting points for this demonstration.

# 24. Regulatory / Policy Considerations

The discussion explicitly noted that net neutrality and related legal/policy considerations must be evaluated.

Because the concept potentially differentiates network treatment based on application/service identification, policy and legal review will be necessary before production deployment.

The meeting also referenced previous enterprise QoS/service-treatment mechanisms that had apparently undergone legal consideration, suggesting there may be useful precedent, although the proposed new mechanism is more sophisticated.

# 25. Key Technical Risks and Open Questions

The following issues remain open and should drive the next technical discussions:

- Core-to-RAN interface: How exactly will UPF classification information be transported to the RAN?

- Information model: What information should be sent—application, service, sub-application, flow identity, priority recommendation, or another abstraction?

- Granularity: Should information be associated with a UE, session, QoS flow, IP flow, or individual service?

- Update frequency: How often must service information be communicated for RAN actions to remain useful?

- RAN action: What specific scheduler/resource-management action should be triggered?

- Multi-vendor interoperability: How will the mechanism behave with Nokia, small cells, roaming and other RAN environments?

- Encryption: How reliably can classification continue as QUIC/TLS privacy mechanisms evolve?

- VPN traffic: How accurately can FirstNet applications be classified when traffic is encapsulated inside VPN tunnels?

- FirstNet policy: Exactly which applications/services should receive uplift, normal treatment or down-prioritization?

- Regulatory compliance: How should net-neutrality and other legal requirements constrain application-aware treatment?

# 26. Agreed Direction / Next Steps

The technical direction emerging from the discussion was:

Step 1 — Demonstrate existing UPF classification capability.

Use test UE traffic and analytics to show that the UPF can identify applications/services.

Step 2 — Establish a traffic baseline.

Analyze actual network traffic categories to determine where meaningful optimization opportunities exist.

Step 3 — Define two concrete FirstNet use cases.

Use one uplift case and one down-prioritization case to drive architecture development.

Step 4 — Define the Core-to-RAN information.

Determine exactly what service-classification information the RAN needs.

Step 5 — Define the transport/signaling mechanism.

Investigate GTP-U or another suitable mechanism for carrying service-awareness information.

Step 6 — Define RAN behavior.

Determine how the RAN scheduler and QoS/resource-management functions should consume the information.

Step 7 — Address multi-vendor and VPN cases.

These should be treated as architecture requirements, not afterthoughts.

Step 8 — Build the October end-to-end demonstration.

Demonstrate not only service classification but also actual differentiated RAN treatment.

# 27. Overall Technical Conclusion

The most important conclusion from the meeting is that UPF traffic classification itself is already relatively mature. The UPF can use heuristics, signatures, packet inspection and flow-level analysis to classify applications and, in some cases, sub-application services. The existing capability can also identify AI-related applications and is continuously updated as Internet traffic evolves.

The real innovation opportunity—and the primary unresolved engineering problem—is therefore:

UPF Service Intelligence → Core-to-RAN Signaling → RAN Service Awareness → Differentiated Radio Treatment

The success of the concept will depend less on proving that the UPF can classify traffic and more on determining what information is useful to the RAN, how quickly and granularly it should be delivered, what RAN action it should trigger, and whether the complete solution remains practical across multi-vendor, encrypted and VPN environments.