# Technical Meeting Minutes

## UPF Service Classification and Core-to-RAN Service Awareness

### 1. Meeting Purpose

The meeting focused on reviewing the existing service and traffic-classification capabilities available in the UPF and determining how these capabilities could be extended into an end-to-end Core-to-RAN service-aware architecture.

The immediate objective was to establish a common technical understanding of what information the UPF can currently detect and classify. The longer-term objective is to determine how this information can be communicated toward the RAN and used to provide differentiated radio treatment.

The proposed end-to-end concept is:

UE/Application Traffic → UPF Detection and Classification → Service Insight → RAN → Scheduler/QoS Treatment → Differentiated User Experience

## 2. Existing UPF Traffic-Classification Capability

The UPF already performs substantially more functionality than basic packet forwarding.

The high-level processing sequence discussed was:

Incoming Traffic → Packet Detection → Traffic Classification → Policy/Service Processing → Forwarding

The UPF can identify subscriber/session traffic using Packet Detection Rules (PDRs) and information including:

- Source interface

- UE IP address

- Tunnel identifier

- Network instance

- Flow/filter information

The control-plane relationship was described conceptually as:

PCF / Policy → SMF → UPF

Once the appropriate session and traffic flow are identified, the UPF can classify the application or service and apply the associated processing rules.

## 3. UPF Classification Engine

The UPF classification engine does not rely on a single mechanism.

The classification capability combines:

Heuristics + Packet Inspection + Signatures/Patterns + Protocol Information + Classification Rules

Inspection can use information available at different protocol layers, including Layer 3 and Layer 4 characteristics.

Application signatures and traffic patterns are continuously developed and updated so the classification engine can recognize changes in Internet applications and traffic behavior.

The discussion indicated that classification is already a mature UPF capability and supports a large number of applications.

## 4. Application and Sub-Application Classification

A significant capability discussed during the meeting is classification beyond the basic application level.

For example, simply determining:

Application = Facebook

may not provide sufficient information.

A Facebook session could contain multiple independent flows, such as:

Facebook → Messaging

Facebook → Video Streaming

Facebook → Video Call

Facebook → Audio Call

The UPF classification capability can potentially operate at IP-flow granularity and distinguish these different services within the same application.

This granularity is particularly important when application identification will subsequently influence QoS or RAN treatment.

## 5. AI Traffic Classification

The team specifically discussed whether the UPF could distinguish AI traffic from non-AI traffic.

The classification mechanism is being extended with signatures associated with AI-related applications and services. Examples discussed included OpenAI-related traffic and Copilot.

Conceptually:

User Traffic

→ Traditional Web/Application Traffic

→ Video/Streaming Traffic

→ Social Media Traffic

→ AI Application Traffic

The ability to identify AI traffic could eventually support a broader AI-aware network architecture in which AI workloads receive treatment based on their application and network requirements.

No specific policy for differentiated AI traffic treatment was finalized during this meeting.

## 6. Encrypted Traffic Classification

Encryption was identified as an important technical challenge.

The meeting specifically discussed traffic using:

- QUIC over UDP

- TLS over TCP

The classification engine can use observable protocol characteristics and available connection-establishment information, including information associated with TLS ClientHello.

However, application providers are increasingly encrypting additional protocol and header information.

Therefore, the effectiveness of traffic classification must continuously evolve as QUIC, TLS and other privacy/security mechanisms evolve.

## 7. Heuristics and Machine-Learning-Based Signature Development

The vendor described an offline process for continuously improving application classification.

The conceptual workflow is:

Internet/Application Traffic Capture

↓

Offline Lab Analysis

↓

Machine-Learning / Pattern Analysis

↓

New or Updated Application Signatures

↓

Updated Heuristic Package

↓

UPF Classification Engine

Large traffic captures can be analyzed outside the production UPF to discover new patterns and update classification signatures.

This allows the classification library to evolve as new applications appear or existing applications change their traffic characteristics.

# 8. Main Innovation: Core-to-RAN Service Awareness

The meeting established an important distinction:

UPF traffic classification already exists.

The primary new engineering challenge is:

How can the service intelligence generated by the UPF be communicated to the RAN and used effectively?

The proposed architecture is:

UPF

Application / Service / Sub-Service Classification

↓

Core-to-RAN Service Information

↓

RAN

↓

Scheduler / Resource Allocation / QoS Decision

The RAN could potentially use this additional information to make more granular radio-resource decisions.

However, the exact RAN behavior has not yet been defined.

# 9. Core-to-RAN Information Requirements

Several fundamental questions remain unresolved.

The team needs to determine exactly what information should be communicated from the Core toward the RAN.

Potential information includes:

- Application identification

- Service category

- Sub-application identification

- Flow identification

- Priority indication

- QoS recommendation

The required granularity also needs to be determined.

Possible levels include:

Per UE

Per PDU Session

Per QoS Flow

Per IP Flow

Per Application

Per Sub-Application

The information should be sufficiently granular to provide useful RAN intelligence without creating excessive signaling or processing overhead.

# 10. Update Frequency

Another important issue is how frequently the UPF should communicate classification information.

The classification engine may inspect the initial packets of a new traffic flow to determine the application/service.

After the traffic has been classified, subsequent packets can follow a faster processing path.

Therefore, service-awareness information should not necessarily need to be transmitted on every packet.

The architecture needs to determine:

When classification information should be generated → When it should be sent to the RAN → When it should be refreshed or removed

This timing must be sufficiently fast for the RAN to make meaningful scheduling decisions.

# 11. Potential GTP-U Extension

One possible approach discussed was carrying the service information through an additional/custom attribute or information element associated with GTP-U.

Conceptually:

UPF

↓

GTP-U + Service Information

↓

gNB/RAN

However, the meeting clearly established that this capability has not yet been implemented.

The Core and RAN teams still need to determine:

- What information should be carried

- What information element or extension should be used

- How the information should be transported

- How the RAN should interpret it

- What action should be triggered after receiving it

Therefore:

UPF classification = existing capability

while

UPF-to-RAN service-awareness signaling = work in progress

# 12. RAN Processing and Scheduler Behavior

The RAN-side behavior remains one of the most important open technical areas.

Once the RAN receives service information, it must determine how that information should influence radio-resource management.

Potential actions discussed include:

- Scheduler differentiation

- Resource prioritization

- QoS differentiation

- More efficient radio-resource utilization

- Improved treatment of priority traffic

- Improved user experience

The RAN team needs to define the relationship between:

Service Classification → RAN Policy → Scheduler Action

Simply providing application information to the RAN does not itself create value unless that information results in a useful and practical RAN action.

# 13. Multi-Vendor RAN Interoperability

A major architecture concern is the multi-vendor nature of the RAN.

The same Core can serve:

- Ericsson RAN

- Nokia RAN

- Small cells

- Other RAN implementations

If a proprietary Core-to-RAN service-awareness mechanism is introduced, the solution must ensure that unsupported RAN nodes are not negatively affected.

A potential architecture is:

UPF

↙　　　　　　　　　↘

Supported RAN　　　 Unsupported RAN

Service-awareness　　　 Standard behavior

information enabled　　 No proprietary information

The Core may therefore need the ability to determine whether a destination RAN supports the feature before inserting or sending the additional information.

# 14. Multi-Vendor Operational Risk

A previous premium/turbo-type service was discussed as an example of the potential operational problem.

The service worked correctly in one RAN environment but did not receive equivalent treatment after the subscriber moved into another vendor's small-cell environment.

This demonstrated that a service feature cannot be designed only for one radio environment.

The new architecture must explicitly consider:

- Mobility between RAN vendors

- Small-cell environments

- Roaming

- Feature capability discovery

- Selective feature activation

- Fallback behavior

These requirements should be addressed during architecture development rather than after implementation.

# 15. FirstNet as a Primary Use Case

FirstNet was identified as a strong candidate for demonstrating the value of service-aware treatment.

A FirstNet subscriber may simultaneously generate:

Mission-Critical Traffic

and

Ordinary Consumer/Application Traffic

For example:

FirstNet UE

→ Mission-critical application

→ YouTube

→ Facebook

→ Netflix

→ Other applications

The proposed concept therefore moves beyond:

FirstNet Subscriber → Prioritize Everything

toward:

Subscriber Identity + Application/Service Context → Appropriate Network Treatment

This would potentially allow the network to differentiate traffic within the same subscriber session.

# 16. Importance of Fine-Grained FirstNet Classification

The team emphasized that broad classifications such as "video" or "streaming" are insufficient.

For example, YouTube should not automatically be classified as entertainment traffic, because first responders may use YouTube and other social-media services for operational or investigative purposes.

Similar considerations apply to:

- Facebook Live

- Instagram

- Other social/video platforms

Therefore:

Video → Deprioritize

would be too simplistic.

Instead, classification should distinguish the actual application and, where required, the sub-application or individual flow.

# 17. FirstNet Use Case 1 – Down-Prioritization

The first proposed use case involves identifying selected entertainment applications and potentially reducing their priority for an appropriate subscriber category.

Applications discussed included:

- Netflix

- Amazon Prime

- Hulu

The concept is:

FirstNet Extended-Primary User

↓

UPF detects selected entertainment application

↓

Service classification communicated

↓

Lower-priority treatment

The discussion indicated that the initial target would not be primary FirstNet subscribers but potentially extended-primary/subscriber-paid users.

YouTube and Facebook should not automatically receive this treatment because they may have legitimate public-safety uses.

# 18. FirstNet Use Case 2 – Mission-Critical Uplift

The second use case operates in the opposite direction.

A push-to-talk/LMR interworking gateway was discussed that allows interoperability between mobile devices and LMR radios.

The current challenge is assigning the desired mission-critical QoS treatment to traffic associated with this type of device.

The proposed flow is:

LMR Gateway Traffic

↓

UPF Detects Device/Service

↓

Mission-Critical Service Recognized

↓

Priority/QoS Treatment Applied

This represents the uplift use case.

The two FirstNet cases therefore provide complementary demonstrations:

Use Case 1: Down-prioritize selected non-critical traffic

Use Case 2: Uplift identified mission-critical traffic

# 19. VPN Traffic Challenge

VPN traffic was identified as an important issue, particularly for FirstNet and public-safety applications.

A significant amount of public-safety traffic may operate inside VPN tunnels.

The resulting classification problem is:

Original Application

↓

VPN Encryption / Encapsulation

↓

UPF

↓

Application Classification?

Once traffic is encapsulated inside a VPN, the UPF may have significantly reduced visibility into the original application.

Therefore, the team needs to evaluate:

- What VPN traffic can currently be identified

- What metadata remains visible

- Classification accuracy inside VPN environments

- Whether the proposed FirstNet use cases remain practical when VPNs are used

# 20. Need for Real Network Traffic Baseline

Before selecting optimization policies, the team proposed analyzing the actual distribution of application traffic in the network.

Available analytics/EBM information could potentially provide a traffic breakdown such as:

Network Traffic

→ YouTube: X%

→ Meta: Y%

→ Netflix: Z%

→ AI traffic: A%

→ Other: B%

The purpose is to identify where meaningful optimization opportunities actually exist.

Rather than designing service-aware policies based only on theoretical applications, the team should first determine which traffic categories represent significant portions of real network usage.

# 21. Near-Term Core/UPF Demonstration

The near-term demonstration is intended to focus on capabilities already available in the Core/UPF.

The planned demonstration flow is:

Generate Test UE Traffic

↓

UPF Receives Traffic

↓

Classification Engine Identifies Application/Service

↓

Analytics/EA Collects Classification Information

↓

Dashboard Displays Classified Traffic

Applications such as YouTube and Netflix were discussed as examples.

The important scope clarification is:

This demonstration is focused on the Core/UPF classification capability.

It is not yet an end-to-end RAN differentiation demonstration.

Its purpose is to establish a common understanding of what service information can already be produced by the UPF.

# 22. October End-to-End Demonstration Target

The subsequent October target is intended to move beyond simply demonstrating application identification.

The target architecture is:

UE/Application

↓

UPF Classification

↓

Service Information

↓

Core-to-RAN Signaling

↓

RAN Service Awareness

↓

Scheduler / QoS Action

↓

Observable Differentiated Treatment

The proposed FirstNet uplift and down-prioritization use cases were identified as potential drivers for this end-to-end demonstration.

# 23. Regulatory and Policy Considerations

The team noted that service-aware differentiated treatment must also consider regulatory and policy requirements, including net-neutrality considerations.

Application-aware traffic differentiation may therefore require legal and policy review before production implementation.

This should be considered alongside the technical architecture rather than only after the solution has been completed.

# 24. Key Technical Open Questions

The following technical questions remain open:

- Core-to-RAN interface: How will UPF service-classification information be communicated to the RAN?

- Information model: What information should be communicated—application, sub-application, service class, flow identity, priority or QoS recommendation?

- Granularity: Should service awareness operate per UE, PDU session, QoS flow, IP flow, application or sub-application?

- Update frequency: How frequently must classification information be sent for the RAN to use it effectively?

- RAN behavior: What specific scheduler or resource-management action should be triggered?

- Multi-vendor interoperability: How will the feature behave with Ericsson RAN, Nokia RAN, small cells and roaming scenarios?

- Encryption: How will evolving QUIC/TLS encryption affect classification accuracy?

- VPN support: How effectively can application traffic be classified when encapsulated inside VPN tunnels?

- FirstNet policy: Which applications should receive priority uplift, normal treatment or down-prioritization?

- Regulatory requirements: What constraints apply to application-aware traffic differentiation?

# 25. Agreed Technical Direction and Next Steps

### Step 1 – Demonstrate Existing UPF Classification

Use test UE traffic to demonstrate existing application and service identification capabilities.

### Step 2 – Establish Network Traffic Baseline

Use analytics/EBM information to understand actual application traffic distribution.

### Step 3 – Finalize FirstNet Use Cases

Define one uplift and one down-prioritization use case as concrete engineering examples.

### Step 4 – Define Required RAN Information

Determine exactly what service information is useful to the RAN.

### Step 5 – Define Core-to-RAN Signaling

Investigate GTP-U or another appropriate mechanism for transporting service-awareness information.

### Step 6 – Define RAN Actions

Determine how the scheduler, QoS and radio-resource-management functions should use the information.

### Step 7 – Address Multi-Vendor, Encryption and VPN Scenarios

These should be treated as fundamental architecture requirements.

### Step 8 – Develop End-to-End Demonstration

Move from:

Application Classification

to:

Application Classification → Core-to-RAN Service Awareness → Differentiated RAN Treatment

# 26. Technical Conclusion

The meeting established that UPF service classification is already a relatively mature capability. The UPF can combine heuristics, signatures, packet inspection, protocol information and flow-level analysis to identify applications and, in some cases, sub-application services.

The principal engineering challenge is therefore not simply:

"Can the UPF identify the application?"

The more important question is:

"How can the network convert UPF service intelligence into useful RAN behavior?"

The overall technical direction can be summarized as:

UPF Service Intelligence

↓

Core-to-RAN Signaling

↓

RAN Service Awareness

↓

Scheduler / QoS Decision

↓

Differentiated Radio Treatment

The next phase should focus on determining the information model, signaling mechanism, granularity, timing and RAN action, while ensuring that the solution remains practical in multi-vendor, encrypted and VPN environments.