Technical Meeting Minutes

UPF Service Classification and Core-to-RAN Service Awareness

1. Meeting Objective

The meeting reviewed the current traffic and service-classification capabilities available within the UPF and discussed how these capabilities could be extended into an end-to-end Core-to-RAN service-aware architecture.

The immediate objective is to establish a common understanding of what application and service information the UPF can currently identify. The longer-term objective is to determine how this service intelligence can be communicated to the RAN and translated into differentiated radio treatment.

The overall concept discussed was:

UE/Application Traffic → UPF Detection and Classification → Service Information → RAN → Scheduler/QoS Treatment → Differentiated User Experience

2. Existing UPF Traffic Classification

The UPF performs considerably more than basic packet forwarding. Its processing can conceptually be represented as:

Incoming Traffic → Packet Detection → Traffic Classification → Policy/Service Processing → Forwarding

Packet Detection Rules (PDRs) and associated session information can be used to identify subscriber and session traffic. Relevant information may include:

- Source interface

- UE IP address

- Tunnel identifier

- Network instance

- Flow/filter information

The control-plane relationship was described as:

PCF / Policy → SMF → UPF

After identifying the appropriate session and traffic flow, the UPF can perform application/service classification and apply the corresponding processing rules.

3. UPF Classification Engine

The UPF classification engine does not depend on a single identification method. Instead, classification can combine:

Heuristics + Packet Inspection + Signatures/Patterns + Protocol Information + Classification Rules

Inspection may use observable information from different protocol layers, including Layer 3 and Layer 4 characteristics.

Application signatures and traffic patterns are continuously developed and updated to account for changes in Internet applications and traffic behavior.

The discussion established that application classification is already a relatively mature UPF capability and supports identification of a large number of applications.

4. Application and Sub-Application Classification

An important point discussed was the ability to classify traffic at a level finer than simply identifying the application.

For example, identifying:

Application = Facebook

may not provide enough information because the application may generate several independent types of traffic:

Facebook → Messaging
Facebook → Video Streaming
Facebook → Video Call
Facebook → Audio Call

The UPF classification capability can potentially operate at IP-flow granularity and distinguish different services or sub-applications associated with the same application.

This becomes particularly important when classification information is intended to influence QoS or RAN scheduling decisions.

5. AI Traffic Classification

The team discussed whether UPF classification could distinguish AI traffic from non-AI traffic.

The classification capability is being extended with signatures associated with AI-related applications and services. OpenAI-related traffic and Copilot were discussed as examples.

Conceptually:

User Traffic
→ Traditional Web/Application Traffic
→ Video/Streaming Traffic
→ Social Media Traffic
→ AI Application Traffic

AI application identification could eventually contribute to a broader AI-aware network architecture in which AI workloads receive treatment according to their application and network requirements.

However, no specific differentiated policy for AI traffic was finalized during the meeting.

6. Classification of Encrypted Traffic

Encryption was identified as an important technical challenge.

The discussion specifically addressed:

- QUIC over UDP

- TLS over TCP

The classification engine can use observable protocol characteristics and available connection-establishment information, including information associated with TLS ClientHello.

However, application providers continue to encrypt additional protocol and header information. Consequently, classification techniques must continuously evolve as QUIC, TLS, and other privacy and security mechanisms evolve.

7. Heuristics and Machine-Learning-Based Signature Development

The vendor described an offline mechanism used to improve application classification continuously.

The conceptual process is:

Internet/Application Traffic Capture
↓
Offline Lab Analysis
↓
Machine-Learning / Pattern Analysis
↓
New or Updated Application Signatures
↓
Updated Heuristic Package
↓
UPF Classification Engine

Large traffic captures can therefore be analyzed outside the production UPF to identify new traffic patterns and develop updated application signatures.

This allows the classification library to evolve as applications and their network behavior change.

8. Main Engineering Focus – Core-to-RAN Service Awareness

A major distinction was es